CN LAN MST

ANS 1:

- Create the network topology with routers, switches, and PCs.

- Assign IP addresses to all router interfaces and PCs.

- Configure the subnet mask for each interface.

- Enable the router interfaces using no shutdown.

- Check connectivity between directly connected networks using ping.

- Identify the destination network that is not directly connected to each router.

- Repeat the static route configuration on the other routers as required.

- Test end-to-end connectivity between PCs

Ans 2:

Answer 2) VLAN Configuration

Step 1: Connect 6 PCs and 1 switch in the topology as shown in the picture.

Step 2: Assign IP addresses to all the PCs according to the given topology.

Step 3: Create VLANs HR, ADMIN, and Finance with VLAN IDs 10, 20, and 30 respectively using the following commands in privileged configuration mode:

vlan <VLAN ID>

name <VLAN Name>

For example:

vlan 10

name HR

Step 4: Assign HR VLAN (10) to Fa0/1 and Fa0/2 using:

interface fa0/1

switchport mode access

switchport access vlan 10

Repeat the same for Fa0/2.

Step 5: Assign ADMIN VLAN (20) to Fa0/3 and Fa0/4 using:

interface fa0/3

switchport mode access

switchport access vlan 20

Repeat the same for Fa0/4.

Step 6: Assign Finance VLAN (30) to Fa0/5 and Fa0/6 using:

interface fa0/5

switchport mode access

switchport access vlan 30

Repeat the same for Fa0/6.

Step 7: Configure Fa0/24 as a trunk port so that it can carry traffic from all the VLANs:

interface fa0/24

switchport mode trunk

Step 8: Verify that all three VLANs have been created using:

show vlan brief

Step 9: Verify that the respective interfaces are assigned to the correct VLANs and that Fa0/24 is in trunk mode using:

show interfaces trunk

Step 10: Test the network connectivity using ping between the PCs and verify that the VLAN configuration is working correctly.